# SmartSense™ Pro Gateway

## Detailed Technical Specification & Requirement Freeze Document

## 1. Introduction & Scope

### 1.1 Purpose of This Document

This document defines the complete, detailed, and binding technical specification of the SmartSense™ Pro Gateway. It is intended to eliminate ambiguity between parties and serve as the sole reference for: - hardware design and validation - firmware and software development - integration with sensors, CAN, mobile application, and cloud - testing, acceptance, and change management

Upon formal approval, this document establishes a Requirement Freeze. Any deviation thereafter shall follow the Change Control procedure defined herein.

### 1.2 Product Definition

SmartSense™ Pro Gateway is an automotive-grade BLE + LTE gateway for RV and trailer environments. The device: - collects data from BLE TPMS sensors and other defined sensors - reads selected RV-C CAN parameters (read-only) - acquires GNSS time and location - monitors motion and vibration - securely reports data to a cloud backend via LTE - exposes configuration, diagnostics, and real-time data via BLE to a mobile application

### 1.3 In Scope

- SmartSense™ Pro Gateway hardware (Board v1)

- Gateway firmware (ESP32-S3 based)

- BLE TPMS sensor integration (Schrader / Sensata AirCheck)

- BLE mobile application interface

- LTE cloud communication via MQTT over TLS

- OTA update mechanism for gateway firmware

- Testing, acceptance, and change control

### 1.4 Out of Scope

The following are explicitly out of scope: - SmartSense Basic, Plus, SignalBoost products - BLE Mesh, relays, repeaters, or multi-hop networking - Wi‑Fi connectivity - CAN actuation or ECU write operations - Multi-gateway coordination - OTA updates for sensors

### 1.5 Definitions & Abbreviations

- BLE – Bluetooth Low Energy

- TPMS – Tire Pressure Monitoring System

- RV‑C – Recreational Vehicle CAN protocol

- OTA – Over‑the‑Air Update

- GNSS – Global Navigation Satellite System

## 2. System Overview

### 2.1 Functional Overview

The SmartSense™ Pro Gateway operates as a central data aggregation, evaluation, and reporting unit within an RV environment. In addition to sensor monitoring, the gateway performs derived computations such as odometer tracking based on GNSS data and event logging based on accelerometer and system state.

### 2.2 Sensor Model & Limits

- Maximum supported TPMS sensors: 6

- Sensors are identified by static BLE MAC address

- No classical BLE pairing is required for TPMS sensors

- Sensor-to-position mapping is configured via the mobile application

### 2.3 Derived Functions

#### 2.3.1 GNSS-based Odometer

- The gateway shall compute total traveled distance (odometer) using GNSS position fixes.

- Distance calculation shall be based on successive valid GNSS fixes using great-circle distance.

- Odometer accumulation rules:

  - Accumulation only when GNSS fix is valid

  - Minimum movement threshold applied to suppress GNSS jitter

  - Distance accumulated in meters

- Odometer value shall:

  - be persistent across reboots

  - be readable via BLE and LTE

  - be included in reporting if enabled in the data model

#### 2.3.2 G-Event Logging

- The gateway shall log significant G events detected by the accelerometer.

- G events are defined as acceleration exceeding configured thresholds on any axis.

- For each G event, the following shall be logged:

  - timestamp (GNSS time)

  - axis (X/Y/Z)

  - peak acceleration value

  - duration above threshold

- G event logs shall be stored locally and reported via LTE according to reporting policy.

## 3. System Architecture

### 3.1 Logical Architecture

- BLE Central role: Gateway → TPMS sensors

- BLE Peripheral role: Gateway → Mobile application

- LTE Client: Gateway → Cloud backend

- CAN Host: Gateway → RV‑C bus (read-only)

- GNSS Receiver: Time and location source

### 3.2 Data Flow

- TPMS sensors broadcast BLE advertising frames

- Gateway scans continuously and filters by configured sensor MAC addresses

- Advertising payloads are decrypted and parsed

- Parsed data is stored locally and evaluated against alarm thresholds

- Data is reported via:

  - BLE (on-demand, synchronous)

  - LTE (periodic, event-driven, and on-demand)

## 4. Hardware Specification – Board v1

### 4.1 MCU / Controller

- Module: ESP32-S3-WROOM-1U-N16R8

- Flash: 16 MB

- PSRAM: 8 MB

- Operating temperature: –40°C to +85°C

- Interfaces used:

  - UART (LTE modem)

  - SPI (CAN controller)

  - I2C (accelerometer)

  - ADC (external voltage measurement)

  - USB (native)

### 4.2 Programming & Debug

- Programming header: GND, EN, RX, TX, GPIO0

- USB: D+ / D− routed to USB‑C connector

- Usage: factory flashing, factory test firmware, service diagnostics

### 4.3 Cellular Modem

- Module: Quectel EG916Q-GL

- LTE categories and bands supported (per module specification):

  - LTE-FDD: B1, B2, B3, B4, B5, B7, B8, B12, B13, B14, B18, B19, B20, B25, B26, B28, B66

  - LTE-TDD: B38, B39, B40, B41

- Operating temperature: –40°C to +85°C

- Antenna connector: u.FL

- Module: Quectel EG916Q‑GL

- LTE connectivity as per module specification

- Operating temperature: –40°C to +85°C

- Antenna connector: u.FL

### 4.4 GNSS

- Module: LC29HAA

- Supported constellations: GPS (per module spec)

- Operating temperature: –40°C to +85°C

- Antenna: u.FL external or patch antenna

- GNSS is the authoritative time source for the system

### 4.5 CAN Interface

- CAN controller: MCP2515

- Operating temperature: –40°C to +125°C

- CAN transceiver and protection as per Nik CAN converter reference design

- Protocol: RV‑C only

- Direction: strictly read-only

### 4.6 Accelerometer

- Sensor: Bosch BMA400

- Operating temperature: –40°C to +85°C

- Interface: I2C

- Used for motion detection and XYZ alarm thresholds

### 4.7 Power Supply

- Nominal input voltage: 12 V vehicle power

- Supported input range: 9–32 V

- Power protection features:

  - Reverse polarity protection

  - Load dump and transient suppression (automotive transients)

  - Over-voltage protection

  - Under-voltage detection

- External supply voltage measured via MCU ADC

- Wake-up control:

  - Digital wake input asserted when voltage > 10 V

  - Deasserted when voltage ≤ 10 V

- Brown-out behavior:

  - Graceful shutdown when voltage drops below threshold

  - State preserved according to persistence rules

### 4.8 User Interface

- Button: 1× physical button (service / reset)

- LEDs:

  - Green: Power present

  - Red: LTE / GSM status

  - Blue: Controller / system status

### 4.9 External Connectors

- Power input: 2 positions

- CAN H: 2 positions

- CAN L: 2 positions

### 4.10 External Connectors

Electrical Scheme of the device is attached to the current document as Annex J

## 5. BLE Specification

### 5.1 BLE Roles

- Simultaneous operation as:

  - BLE Central (TPMS sensors)

  - BLE Peripheral (mobile app)

### 5.2 TPMS Sensor Protocol

TPMS sensors comply with the Schrader / Sensata AirCheck BLE TPMS specification. The gateway shall: - support BLE v4.2 or later - decode advertising and connected-mode frames - decrypt AES‑128 ECB encrypted payloads (version 0x02)

Decoded parameters include: - Pressure - Temperature - Battery status - Status and fault flags - Firmware version - Block counter

### 5.3 Alarm Handling

The gateway shall generate alarms based on decoded TPMS data, including: - Low pressure - High pressure - High temperature - Sensor low battery - Sensor internal fault - Sensor failure (missing advertisements)

### 5.4 BLE GATT Specification

Complete GATT specification of Smart Sense Gateway is attached to the current document as Annex 2 – BLE GATT Specification

## 6. Data Model (Normative)

### 6.1 Normative Source

The provided Excel database is the single source of truth for all parameters exposed by the gateway over BLE and LTE. - Column L defines semantics (Command / Measurement / Setting). - Column M defines whether the parameter is included in the Standard Report.

### 6.2 Parameter Definition Template (Normative)

Each parameter shall be represented by one row in the table attached Annex 1 – Data Model / Parameter Registry. The populated tables are normative and override any conflicting interpretation elsewhere in this document.

### 6.3 TPMS Parameters (Per-Wheel)

The gateway supports up to 6 TPMS sensors, each mapped explicitly to a wheel index (1…6). All TPMS parameters are defined per wheel. Wheel index to physical position mapping is configured via the mobile application.

### 6.4 Environmental Parameters (CAN-sourced)

All environmental measurements (temperature, humidity) are sourced exclusively from RV-C CAN. No local environmental sensors are present in the gateway.

### 6.5 CAN System & RV-C Parameters

This section defines gateway-level CAN/RV-C behavior parameters. These parameters do not represent physical sensor measurements, but control and describe how the gateway interacts with the RV-C bus.

#### CAN Safety Rules (Normative)

- The gateway shall never transmit CAN frames under any condition.

- Any attempt to enable CAN transmission shall be ignored.

- CAN actuation is explicitly forbidden in this product.

### 6.6 Configuration & Reporting Parameters

This section defines all system-level configuration parameters controlling logging, reporting cadence, feature enables, and rate limits. These parameters are mandatory for predictable device behavior and acceptance testing.

### 6.7 Reporting Sets

(Template) The following sets shall be defined by the populated table above: - Standard Report Set: all parameters marked Included in Standard Report = Yes. - Low-Power Report Set: all parameters marked Included in Low-Power Report = Yes. - Event Report Set: all parameters marked Event Trigger = Yes.

### 6.8 Read/Write Rules

- If a parameter is W or RW, the write operation shall define:

  - validation rules

  - persistence rule

  - effective time

  - response/ACK payload

## 7. Reporting Policy

### 7.1 Hybrid Reporting Model

The gateway implements a hybrid reporting strategy: - Periodic reporting - Event-driven reporting - On-demand synchronous reporting. On-demand reporting is triggered only after Periodic or Event-driven report.

### 7.2 Periodic Reporting

- Reporting intervals are controlled by configurable parameters:

  - Log period

  - Report period – Standard

  - Report period – Low Power

### 7.3 Event-Driven Reporting

Event-driven reports are generated when alarm conditions are met.

#### 7.3.1 Alarm Enable Logic (Normative)

For every alarm-capable parameter, the following logic applies: - Alarm evaluation is performed only if the corresponding Alarm Enable parameter = 1. - If Alarm Enable = 0, no alarm shall be triggered regardless of measured value.

#### 7.3.2 Threshold Logic

- High alarm is triggered when:

  - Measured value ≥ configured High Threshold

- Low alarm is triggered when:

  - Measured value ≤ configured Low Threshold

#### 7.3.3 Alarm Clear Conditions

- High alarm is cleared when:

  - Measured value < High Threshold

- Low alarm is cleared when:

  - Measured value > Low Threshold

#### 7.3.4 Environmental Alarms

Environmental alarms include, but are not limited to: - Temperature high / low - Humidity high / low - External supply voltage low

Each environmental alarm shall define explicitly: - Enable parameter - High and/or Low threshold parameter - Reporting behavior (event + standard report inclusion)

All alarm driven reports are defined in Annex 3 – Alarm Specification

### 7.4 On-Demand reporting

All the requests for reporting from the cloud to the Gateway must be queued and when the Gateway initiate connection for periodic or event-driven report the cloud must within 2 seconds respond to the downlink message topic with commands for on-demand reporting.

## 8. LTE & Cloud Interface

### 8.1 Protocol

- MQTT over TLS

- Cloud bootstrap procedure for certificate provisioning

### 8.2 Command Handling

- Cloud-to-device commands are supported

- Commands are executed synchronously

- ACK and retry mechanism implemented

### 8.3 Offline Operation

- Minimum offline buffering: 7 days

- Oldest records overwritten first

- Event order preserved

### 8.4 Device Identity

Device Serial Number: MCU Serial Number (ESP32)

Cloud identity is bound to MCU Serial number - hardware uniqueness is guaranteed by the MCU serial number.

### 8.5 Cloud Endpoint

Cloud Platform: AWS IoT Core
MQTT Broker URL: TBD
Transport: MQTT over TLS

### 8.6 MQTT Topics

Uplink (Device → Cloud): ss/u/{SN}
Downlink (Cloud → Device): ss/d/{SN}

### 8.7 MQTT Payload Format

Payloads are JSON dictionaries with key-value pairs:
{ <parameterID>: <value> }

Parameter IDs shall match Annex 1 exactly. Symbolic names are not permitted.

Applicable downlink message commands are:

- GET commands - *GET,{parameter ID}$ - initiate request to read value of device database applicable for parameter types:

  - Setting

  - Measurement

Example: *GET,221$

- SET commands –

  - *SET,{parameter ID},{value}$ – initiate request to write value to device database applicable for parameter type Setting

Example: $SET,221,21$ sets value 21 to parameter 221

  - *SET,{parameter ID}$ - initiate request to execute command to device. Applicable for parameter type Command

Example: $SET,220$ executes command 220

### 8.8 Timestamp Requirement

Each uplink payload shall include Parameter ID 257:
UNIX Timestamp (UTC)

### 8.9 Data Types

0 – String
1 – Float
2 – Boolean
3 – Buffer
9 – Function

### 8.10 Write & Command Semantics

Downlink messages shall contain only writable Parameter IDs and shall be processed synchronously with acknowledgment.

## 8.11 Bootstrapping & Certificate Provisioning

### 8.11.1 Bootstrap Conditions

Bootstrapping is initiated if no valid certificate is present or TLS authentication fails.

### 8.11.2 Bootstrap Procedure

Attempt MQTT connection. On failure, enter bootstrap state, connect to bootstrap URL (TBD), obtain credentials, store securely, and retry MQTT connection.

### 8.11.3 Failure Handling

Bootstrap failures shall not block local operation or erase buffered data.

## 9. CAN Host Integration

- RV‑C protocol only

- Read-only access

- Only parameters defined in the database and corresponding to installed sensors are read

## 10. Firmware & Deliverables

- Full firmware source code delivery

- Build scripts and third-party license disclosures

- Production firmware binary

- Factory test firmware binary

- Full IP ownership transferred to the client

## 11. OTA Update Mechanism

Detailed Over-the-air update mechanism is defined in Annex 4 – OTA Procedures

- OTA initiated via cloud or BLE

- Three-image architecture:

  - Base (factory image)

  - Last (currently running)

  - New (update image)

- Automatic rollback and watchdog recovery

## 12. Security & Privacy

- BLE bonding enabled

- LTE identity based on X.509 certificates

- Certificate provisioning via AWS

- No anonymization required

## 13. Testing & Acceptance

### 13.1 Functional Tests

- Correct decoding and decryption of TPMS advertising frames

- Correct alarm generation and clearing

- Correct BLE and LTE parameter access

### 13.2 Performance Tests

- BLE processing latency < 1 s

- Event detection latency < 1 s

- GNSS time drift ≤ 1 s

### 13.3 RF Tests

- BLE reception stability

- LTE attach and throughput validation

### 13.4 CAN Tests

- RV‑C read-only verification

- No CAN transmission under any condition

### 13.5 OTA Tests

- Successful update

- Forced rollback scenarios

### 13.6 Acceptance Authority

Final acceptance is signed by the client’s Business Owner.

## 14. Change Control & Requirement Freeze

### 14.1 Requirement Freeze Declaration (Normative)

Upon written approval of this document by both parties, all requirements defined herein are frozen. In particular: - Section 6 – Data Model is declared normative and frozen. - Any implementation shall conform exactly to the defined parameters, scaling, access rules, and alarm semantics. - No implicit behavior, undocumented parameter, or inferred functionality shall be considered in scope.

### 14.2 Change Request (CR) Process

Any deviation from this specification after freeze requires a formal Change Request (CR).

Each CR shall include: - Unique CR identifier - Description of requested change - Technical rationale - Affected sections and parameters - Requested priority

### 14.3 Impact Analysis

Each CR shall be evaluated for: - Technical impact (firmware, hardware, cloud, app) - Schedule impact - Cost impact - Regression and validation impact

### 14.4 Approval & Re-pricing

- No change shall be implemented without written approval from the client.

- Approved changes may result in re-pricing and schedule adjustment.

## 15. Acceptance Mapping (Normative)

This section maps specification elements to acceptance criteria to ensure objective verification.

| Spec Section | Acceptance Test Category | Acceptance Method |
| --- | --- | --- |
| Hardware Specification | Hardware inspection & power tests | Bench + environmental tests |
| BLE TPMS Data Model | Functional & decoding tests | BLE test vectors |
| CAN Parameters | CAN protocol tests | RV-C bus simulation |
| Reporting Parameters | Timing & load tests | Cloud + LTE tests |
| Alarm Logic | Threshold tests | Boundary condition testing |
| OTA Mechanism | Update & rollback tests | Forced failure scenarios |

## 16. Editorial Consistency & Terminology

- All parameter names are UPPER_SNAKE_CASE and globally unique.

- Units follow SI conventions unless explicitly stated.

- Time values are expressed in seconds unless otherwise noted.

- Temperature values are expressed in °C.

## Annexes

Annex 1 – Data Model / Parameter Registry - Annex_1_SmartSense_Gateway_Database.xlsx

Annex 2 – BLE GATT Specification - Annex_2_BLE_GATT.pdf

Annex 3 – Alarm Specification - Annex_3_Alert_Mapping.xlsx

Annex 4 – OTA Procedures - Annex_4_OTA_Update_Procedures.pdf

Annex 5 – Device State Chart - Annex_5_State_Chart.pdf

Annex 6 – Test Plan & Test Cases - Annex_6_Test_Plan.pdf

Annex 7 – Electrical Schematic - Annex_7_Electrical_Schematic.pdf

Annex 8 – Bill of Materials- Annex_8_BOM_SmartSensePro.xlsx

Annex 9 - Schrader / Sensata AirCheck BLE TPMS specification – Annex_9_Schrader AirCheck BLE TPMS System Requirements.pdf

End of Document