# Annex A – BLE GATT Specification (Normative)

This annex defines the complete BLE GATT for SmartSense™ Pro Gateway. Services are grouped by the Excel 'Category' column and a characteristic is defined for every row in the Excel database.

## A.1 UUID Scheme (Normative)

Base UUID tail: 1B2C-4F8A-8C7D-9A1B2C3D4E5F

Service UUID format: D972CCCC-1B2C-4F8A-8C7D-9A1B2C3D4E5F where CCCC is the category code.

Characteristic UUID format: D973IIII-1B2C-4F8A-8C7D-9A1B2C3D4E5F where IIII is the Excel ID in hex.

## A.2 Service List (by Category)

Accelerometer → D9720100-1B2C-4F8A-8C7D-9A1B2C3D4E5F

BLE TPMS → D9720101-1B2C-4F8A-8C7D-9A1B2C3D4E5F

Device Info → D9720102-1B2C-4F8A-8C7D-9A1B2C3D4E5F

Device settings → D9720103-1B2C-4F8A-8C7D-9A1B2C3D4E5F

Firmware Update → D9720104-1B2C-4F8A-8C7D-9A1B2C3D4E5F

GPS → D9720105-1B2C-4F8A-8C7D-9A1B2C3D4E5F

Humidity - CAN → D9720106-1B2C-4F8A-8C7D-9A1B2C3D4E5F

Network Information  → D9720107-1B2C-4F8A-8C7D-9A1B2C3D4E5F

Temperature - CAN → D9720108-1B2C-4F8A-8C7D-9A1B2C3D4E5F

## A.2.1 Service: Accelerometer

Service UUID: D9720100-1B2C-4F8A-8C7D-9A1B2C3D4E5F

| Excel ID | Name | Char UUID | BLE Ops | Properties | HiGPS Type | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 277 | Accelerometer X Value | D9730115-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 278 | Accelerometer Y Value | D9730116-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 279 | Accelerometer Z Value | D9730117-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 281 | Accelerometer Max X Value | D9730119-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 283 | Accelerometer Max Y Value | D973011B-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 285 | Accelerometer Max Z Value | D973011D-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 869 | Accelerometer X alarm hight level | D9730365-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 870 | Accelerometer Y alarm hight level | D9730366-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 871 | Accelerometer Z alarm hight level | D9730367-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 872 | Accelerometer alarm enable | D9730368-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |

## A.2.2 Service: BLE TPMS

Service UUID: D9720101-1B2C-4F8A-8C7D-9A1B2C3D4E5F

| Excel ID | Name | Char UUID | BLE Ops | Properties | HiGPS Type | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 815 | TPMS Temperature alarm high level | D973032F-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 816 | TPMS Temperature alarm low level | D9730330-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 817 | TPMS Temperature alarm enable | D9730331-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 818 | TPMS_1 BLE Address | D9730332-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 819 | TPMS_1 Temperature | D9730333-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 820 | TPMS_1 Tyre pressure | D9730334-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 821 | TPMS_1 Firmware Version | D9730335-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 822 | TPMS_1 Battery Level | D9730336-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 823 | TPMS_1 Status | D9730337-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 824 | TPMS_1 Encryption key | D9730338-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 825 | TPMS_Package URI | D9730339-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 826 | TPMS_Execute Update | D973033A-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | command | Write requires validation per Section 6 rules |
| 827 | TPMS_Update State | D973033B-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 828 | TPMS_Update Result | D973033C-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 829 | TPMS_Download Firmware | D973033D-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | command | Write requires validation per Section 6 rules |
| 830 | TPMS sensor to update | D973033E-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 831 | TPMS Tyre pressure alarm high level | D973033F-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 832 | TPMS Tyre pressure alarm low level | D9730340-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 833 | TPMS Tyre pressure alarm enable | D9730341-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 834 | TPMS_2 BLE Address | D9730342-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 835 | TPMS_2 Temperature | D9730343-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 836 | TPMS_2 Tyre pressure | D9730344-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 837 | TPMS_2 Firmware Version | D9730345-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 838 | TPMS_2 Battery Level | D9730346-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 839 | TPMS_2 Status | D9730347-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 840 | TPMS_2 Encryption key | D9730348-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 841 | TPMS_3 BLE Address | D9730349-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 842 | TPMS_3 Temperature | D973034A-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 843 | TPMS_3 Tyre pressure | D973034B-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 844 | TPMS_3 Firmware Version | D973034C-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 845 | TPMS_3 Battery Level | D973034D-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 846 | TPMS_3 Status | D973034E-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 847 | TPMS_3 Encryption key | D973034F-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 848 | TPMS_4 BLE Address | D9730350-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 849 | TPMS_4 Temperature | D9730351-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 850 | TPMS_4 Tyre pressure | D9730352-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 851 | TPMS_4 Firmware Version | D9730353-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 852 | TPMS_4 Battery Level | D9730354-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 853 | TPMS_4 Status | D9730355-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 854 | TPMS_4 Encryption key | D9730356-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 855 | TPMS_5 BLE Address | D9730357-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 856 | TPMS_5 Temperature | D9730358-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 857 | TPMS_5 Tyre pressure | D9730359-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 858 | TPMS_5 Firmware Version | D973035A-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 859 | TPMS_5 Battery Level | D973035B-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 860 | TPMS_5 Status | D973035C-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 861 | TPMS_5 Encryption key | D973035D-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 862 | TPMS_6 BLE Address | D973035E-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 863 | TPMS_6 Temperature | D973035F-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 864 | TPMS_6 Tyre pressure | D9730360-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 865 | TPMS_6 Firmware Version | D9730361-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 866 | TPMS_6 Battery Level | D9730362-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 867 | TPMS_6 Status | D9730363-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 868 | TPMS_6 Encryption key | D9730364-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |

## A.2.3 Service: Device Info

Service UUID: D9720102-1B2C-4F8A-8C7D-9A1B2C3D4E5F

| Excel ID | Name | Char UUID | BLE Ops | Properties | HiGPS Type | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 200 | imei | D97300C8-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 220 | Firmware Version | D97300DC-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 221 | Reboot | D97300DD-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | command | Write requires validation per Section 6 rules |
| 222 | Factory Reset | D97300DE-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | command | Write requires validation per Section 6 rules |
| 290 | Externa power level (mV) | D9730122-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 312 | Get settings | D9730138-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | command | Write requires validation per Section 6 rules |
| 440 | alarm | D97301B8-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 443 | Init Time | D97301BB-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |

## A.2.4 Service: Device settings

Service UUID: D9720103-1B2C-4F8A-8C7D-9A1B2C3D4E5F

| Excel ID | Name | Char UUID | BLE Ops | Properties | HiGPS Type | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 421 | Log period | D97301A5-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 422 | Report period - Standard | D97301A6-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 873 | Report period - Low power | D9730369-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 874 | External power alarm low level | D973036A-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 875 | External power alarm enable | D973036B-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 884 | BLE Pairing key | D9730374-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 885 | VIN Number | D9730375-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |

## A.2.5 Service: Firmware Update

Service UUID: D9720104-1B2C-4F8A-8C7D-9A1B2C3D4E5F

| Excel ID | Name | Char UUID | BLE Ops | Properties | HiGPS Type | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 51 | Package URI | D9730033-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 52 | Execute Update | D9730034-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | command | Write requires validation per Section 6 rules |
| 53 | Update State | D9730035-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | read | measurement | Write requires validation per Section 6 rules |
| 54 | Update Result | D9730036-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement | Write requires validation per Section 6 rules |
| 55 | Download Firmware | D9730037-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | command | Write requires validation per Section 6 rules |
| 57 | Download flow control | D9730038-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Firmware size 3 bytes int , Packet size (chunk only) 2 bytes int, Packet ID 2 bytes int Upload result 1 byte - mApp set to 0x00 (Uploaded), IoT set to 0x01 (Confirmed), 0x02 (Error), |
| 58 | Binary payload | D9730039-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Message ID (packet ID) 2 bytes - Binary x bytes, CRC checksum (CRC-8 Maxim/Dallas Algorithm) 1 bytes (binary Only !!!) |

## A.2.6 Service: GPS

Service UUID: D9720105-1B2C-4F8A-8C7D-9A1B2C3D4E5F

| Excel ID | Name | Char UUID | BLE Ops | Properties | HiGPS Type | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 214 | Satellite Lock Timeout (sec) | D97300D6-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 254 | Latitude | D97300FE-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 255 | Longitude | D97300FF-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 256 | Altitude | D9730100-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 257 | Timestamp | D9730101-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 258 | Speed | D9730102-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 519 | Number of satelites | D9730207-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 601 | GPS Fix time | D9730259-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 876 | Total mileage | D973036C-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 877 | Last 30 days mileage | D973036D-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |

## A.2.7 Service: Humidity - CAN

Service UUID: D9720106-1B2C-4F8A-8C7D-9A1B2C3D4E5F

| Excel ID | Name | Char UUID | BLE Ops | Properties | HiGPS Type | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 249 | Humidity Value | D97300F9-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 250 | Min Humidity Value | D97300FA-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 251 | Max Humidity Value | D97300FB-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 252 | Reset Min and Max Humidity Values | D97300FC-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | command | Write requires validation per Section 6 rules |
| 878 | Humidity alarm high level | D973036E-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 879 | Humidity alarm low level | D973036F-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 880 | Humidity alarm enable | D9730370-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |

## A.2.8 Service: Network Information

Service UUID: D9720107-1B2C-4F8A-8C7D-9A1B2C3D4E5F

| Excel ID | Name | Char UUID | BLE Ops | Properties | HiGPS Type | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 230 | Network Bearer | D97300E6-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 231 | Radio Signal Strength | D97300E7-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 232 | Link Quality | D97300E8-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 234 | Cell ID | D97300EA-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 235 | SMNC | D97300EB-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 236 | SMCC | D97300EC-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 237 | SignalSNR | D97300ED-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 238 | LAC | D97300EE-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 779 | Band | D973030B-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 780 | Timing Advance | D973030C-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |

## A.2.9 Service: Temperature - CAN

Service UUID: D9720108-1B2C-4F8A-8C7D-9A1B2C3D4E5F

| Excel ID | Name | Char UUID | BLE Ops | Properties | HiGPS Type | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 239 | Temperature Value | D97300EF-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 240 | Min Temperature Value | D97300F0-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 241 | Max Temperature Value | D97300F1-1B2C-4F8A-8C7D-9A1B2C3D4E5F | R | Read | measurement |  |
| 242 | Reset Min and Max Temperature Values | D97300F2-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | command | Write requires validation per Section 6 rules |
| 881 | Temperature alarm high level | D9730371-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 882 | Temperature alarm high level | D9730372-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |
| 883 | Temperature alarm enable | D9730373-1B2C-4F8A-8C7D-9A1B2C3D4E5F | W | Write | setting | Write requires validation per Section 6 rules |

## A.3 BLE Advertising Overview

The device shall advertise using BLE Advertising packets only. Scan Response shall NOT be used. No GATT characteristic shall support Notify. The advertising interval shall be fixed to 1 second.

## A.3.1 Manufacturer Specific Data Format

Manufacturer Specific Data shall be encoded as follows:

• Company Identifier Code: 2 bytes
• Payload Version: 1 byte (0x01)
• Device Serial Number: 6 bytes (Big Endian)
• TPMS Wheel Data: 24 bytes (6 wheels × 4 bytes)

Total Manufacturer Data length: 33 bytes

## A.3.2 TPMS Wheel Data Encoding

For each wheel (1…6), data shall be encoded sequentially:

• Pressure: 2 bytes, uint16, Big Endian, unit mbar, 0xFFFF = invalid
• Temperature: 1 byte, int8, value = raw − 40 °C, 0xFF = invalid
• Battery Level: 1 byte, uint8, percent, 0xFF = unknown

## A.3.3 Wheel Ordering Rule

TPMS wheel data shall always be transmitted in fixed order:
Wheel 1 → Wheel 2 → Wheel 3 → Wheel 4 → Wheel 5 → Wheel 6.

Mapping to physical wheel positions is handled by the mobile application.

## A.3.4 Advertising Update Behavior

The advertising payload shall be updated whenever new TPMS data is received or when any TPMS alarm state changes. The latest known values shall always be broadcast.