# Annex 4 – OTA Update Procedures (Normative)

This document defines the complete Over-the-Air (OTA) firmware update procedures for the SmartSense™ Pro Gateway main controller. The procedure is aligned with the LwM2M Firmware Update state machine and applies to both Cloud (LTE) and BLE updates.

*Source OMA-Lightweight machine to machine technical specification

## 1. Firmware Update State & Result Model

OTA uses the following characteristics:

• State – Characteristic 53
• Result – Characteristic 54

### 1.1 Firmware Update State (53)

0 – Idle

1 – Downloading

2 – Downloaded

3 – Updating

4 – Error

### 1.2 Firmware Update Result (54)

0 – Initial / No result

1 – Success

2 – Not enough storage

3 – Out of memory

4 – Connection lost / Timeout

5 – Integrity check failed

6 – Unsupported / invalid image

7 – Update failed

## 2. Cloud OTA (LTE)

Cloud OTA is controlled via MQTT commands and HTTP GET image download.

### 2.1 Control Characteristics

51 – Firmware Package URL (*SET,51,{string})

55 – Download Trigger (*SET,55)

52 – Apply Update Trigger (*SET,52)

### 2.2 Image Size Discovery

The device SHALL determine the firmware image size by performing:
GET <Firmware Package URL>
The response SHALL be a plain integer representing image size in bytes.

### 2.3 Chunk Download

Firmware chunks SHALL be downloaded using:
GET <URL>/<chunk_size>/<n>
Where n is the chunk index starting from 0.

### 2.4 Download Procedure

Server writes URL to characteristic 51 and receives MQTT ACK.

Server issues SET,55 command.

Device sets State=Downloading (53=1).

Device downloads chunks sequentially.

On success, device sets State=Downloaded (53=2).

On failure, device sets State=Error (53=4) and Result (54).

### 2.5 Apply Update

Upon SET,52, the device applies the firmware, reboots, and reports Success or Failure via State (53) and Result (54).

## 3. BLE OTA

BLE OTA uses the same state machine with binary upload over BLE.

### 3.1 Flow Control

Firmware size (3 bytes), Packet size (2 bytes), Packet ID (2 bytes), Upload result (1 byte).

### 3.2 Binary Payload

Message ID (2 bytes), Binary data (x bytes), CRC-8 Maxim/Dallas (1 byte).

### 3.3 BLE OTA Procedure

App writes flow control parameters.

Device sets State=Downloading.

App uploads binary packets sequentially.

Device validates CRC and stores packets.

Device verifies image and sets State=Downloaded or Error.

Apply update follows Cloud OTA logic.

## 4. Command Acknowledgement

Every server-issued OTA command SHALL be acknowledged via MQTT uplink on topic ss/u/{IMEI}, including State (53), Result (54), and timestamp (257).

## 5. Common Rules

OTA SHALL not erase buffered telemetry.

Rollback is mandatory.

Device SHALL remain operational on failure.

State and Result SHALL always be readable.